(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR ALPHAFLEX® PVC CEILING BANNERS

# PART 1	GENERAL

## 1.01	WORK INCLUDED

The work consists of furnishing all labor, materials, accessories and equipment necessary to cover all areas shown on the drawings and specified herein with AlphaFlex ceiling banners.

## 1.02	DELIVERIES AND STORAGE

- Store this product in unopened cartons in a clean, cool, dry interior space. Cartons and product must be protected against marring, water, dirt or damage during delivery and storage.

## 1.03	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	MANUFACTURER

- Acoustical Solutions
3017 Vernon Road
Richmond, VA 23228
Phone: 800-782-5742
Fax: 804-346-8808
E-mail: estimating@acousticalsolutions.com
Website: AcousticalSolutions.com

## 2.02	PRODUCT

- Geometric Shape: Rectangular

- Size: [Custom Width up to 4'] x [Custom Length up to 20']

- Thickness: [2" Thick] [4" Thick]

- Core: 0.75 lb. acoustical fiberglass

- Surface Finish: [Perforated One Side] [Solid Both Sides]

- Color: [To be selected by architect from manufacturer’s standard selection]

- Edge Shape: Natural

- Edge Construction: [Heat Sealed] [Sewn]

Acoustical Performance:

NRC: 1.00 (2" Thick); 1.15 (4" thick)

Fire Rating: Class “A”

Suspension Provision: [Anodized Aluminum Stiffener/Gripper Bars (for hanging with a festoon)] [Washer Plates, painted to match (for direct mount to roof deck)]

# PART 3	EXECUTION

## 3.01	INSTALLATION

- AlphaFlex™ ceiling banners shall be located and spaced as detailed on reflected ceiling plans and/or detailed on the interior elevation drawings and as indicated on room finish schedule.

- Suspend panels from: [Existing Construction] [Cable]

- Attachment Method: [Heavy-duty Plastic Tie] [Coated Wire Tie] [Beam Clamps] [Washer Plates] [Cable]

## 3.02	WORKMANSHIP

- All work shall be completed in a workman-like manner. All panels shall be properly installed and left in a clean, sanitary condition. Contractor shall remove and dispose of shipping containers and debris.

# END OF SECTION